Abdelrahman Mohamed Ali

UVM FIFO

12 May, 2024

| Gmail: | abdelrahmansalby23@gmail.com |
| --- | --- |
| LinkedIn: | Abdelrahman Mohamed |
| GitHub: | Abdelrahman1810 |

Bug report

| Label | Where in code | Explain | How FIX |
| --- | --- | --- | --- |
| invalid_opcode | Line 23 | opcode_reg[3] is out of range | Replace opcode_reg[3] With opcode_reg[1] |
| Blinking Leds | Line42 | Bypass is lead signal, If Bypass then we ignore invalid Cases | The condition of (if): Bug: invalid FIX: invalid and not bupass |
| Checking invaled for out - bypass | Line 56 58 | Bypass is lead signal, If Bypass then we ignore invalid Cases | Checking for bypass first |
| Case to check opcode | Line 65 | Case should check register opcode not input opcode | case(opcode_reg), instead of: case(opcode) |
| opcode == 3’h0 | Line 68 74 | When opcode==0, Should OR not AND | Use (¦) instead of (&) |
| opcode == 3’h1 | Line 78 84 | When opcode==1, Should XOR not OR | Use (^) instead of (¦) |

assign invalid_opcode = opcode_reg[2] & opcode_reg[3]; // Bug assign invalid_opcode = opcode_reg[2] & opcode_reg[0]; // Fix

if (invalid) // Bug if (invalid && !(bypass_A_reg ¦¦ bypass_B_reg)) // FIX

if (invalid) // Bug out <= 0; // Bug else if (bypass_A_reg && bypass_B_reg) out <= (INPUT_PRIORITY == "A")? A_reg: B_reg; else if (bypass_A_reg) out <= A_reg; else if (bypass_B_reg) out <= B_reg; if (bypass_A_reg && bypass_B_reg) out <= (INPUT_PRIORITY == "A")? A_reg: B_reg; else if (bypass_A_reg) out <= A_reg; else if (bypass_B_reg) out <= B_reg; else if (invalid) // FIX out <= 0; // FIX

case (opcode) // Bug case (opcode_reg) // FIX

&A_reg: &B_reg; // Bug out <= &A_reg; // Bug out <= &B_reg; // Bug out <= A_reg & B_reg; // Bug --------------- ¦A_reg: ¦B_reg; // FIX out <= ¦A_reg; // FIX out <= ¦B_reg; // FIX out <= A_reg ¦ B_reg; // FIX ---------------

¦A_reg: ¦B_reg; // Bug out <= ¦A_reg; // Bug out <= ¦B_reg; // Bug out <= A_reg ¦ B_reg; // Bug ^A_reg: ^B_reg; // Fix out <= ^A_reg; // Fix out <= ^B_reg; // Fix out <= A_reg ^ B_reg; // Fix

/// Do file

vlib work

vlog -coveropt 3 +cover +acc {shared_pkg\shared_pkg.sv}

###

vlog -coveropt 3 +cover +acc {Interface\interface.sv}

vlog -coveropt 3 +cover +acc {Interface\ALSU_ref.sv}

###

vlog -coveropt 3 +cover +acc {DUT\ALSU.sv}

vlog -coveropt 3 +cover +acc {DUT\assertion.sv}

###

vlog -coveropt 3 +cover +acc {objects\configration.sv}

vlog -coveropt 3 +cover +acc {objects\sequence_Items\sequenceItem.sv}

vlog -coveropt 3 +cover +acc {objects\sequence_Items\sequenceItem_Valid.sv}

vlog -coveropt 3 +cover +acc {objects\ALSU_sequence\ALSU_reset_sequence.sv}

vlog -coveropt 3 +cover +acc {objects\ALSU_sequence\ALSU_main_sequence.sv}

###

vlog -coveropt 3 +cover +acc {UVM\Test\env\agent\driver\driver.sv}

vlog -coveropt 3 +cover +acc {UVM\Test\env\agent\monitor\monitor.sv}

vlog -coveropt 3 +cover +acc {UVM\Test\env\agent\sequencer\sequencer.sv}

vlog -coveropt 3 +cover +acc {UVM\Test\env\agent\agent.sv}

###

vlog -coveropt 3 +cover +acc {UVM\Test\env\coverage_collector\coverage_collector.sv}

vlog -coveropt 3 +cover +acc {UVM\Test\env\scoreboard\scoreboard.sv}

vlog -coveropt 3 +cover +acc {UVM\Test\env\env.sv}

###

vlog -coveropt 3 +cover +acc {UVM\Test\test.sv}

vlog -coveropt 3 +cover +acc {UVM\top.sv}

#vsim -voptargs=+acc work.top -classdebug -uvmcontrol=all

vsim +UVM_VERBOSITY=UVM_LOW -voptargs=+acc work.top -classdebug -uvmcontrol=all -cover

add wave -position insertpoint sim:/top/intf/*

add wave -radix hexadecimal sim:/top/intf/out

add wave -radix hexadecimal sim:/top/intf/out_ref

add wave -color Orchid -radix hexadecimal sim:/top/intf/leds

add wave -color gold   -radix hexadecimal sim:/top/intf/leds_ref

run -all

#quit -sim

/// shared_pkg

`timescale 1ps/1ps

package shared_pkg;

import uvm_pkg::*;

`include "uvm_macros.svh"

parameter INPUT_PRIORITY = "A";

parameter FULL_ADDER = "ON";

typedef enum reg [2:0] {OR, XOR, ADD, MULT, SHIFT, ROTATE, INVALID_6, INVALID_7} opcode_e;

parameter MAXPOS = 3;

parameter ZERO = 0;

parameter MAXNEG = -4;

parameter HOBBIT_LOOP = 1000;

parameter DWARF_LOOP = 10_000;

parameter HUMAN_LOOP = 20_000;

parameter GIANT_LOOP = 30_000;

bit alwaysValid = 0;

int RESET_DIST = 5;

endpackage

/// interface

`timescale 1ps/1ps

import shared_pkg::*;

interface ALSU_interface(input bit clk);

logic rst, cin, red_op_A, red_op_B, bypass_A, bypass_B, direction, serial_in;

opcode_e opcode;

logic signed [2:0] A, B;

logic [15:0] leds;

logic [5:0] out;

logic [15:0] leds_ref;

logic [5:0] out_ref;

bit inputs_are_driven;

endinterface //ALSU_interface

/// ALSU_ref

import shared_pkg::*;

module ALSU_ref (A, B, cin, serial_in, red_op_A, red_op_B, opcode, bypass_A, bypass_B, clk, rst, direction, leds, out);

input clk, rst, cin, red_op_A, red_op_B, bypass_A, bypass_B, direction, serial_in;

//input opcode_e opcode;

input [2:0] opcode;

input signed [2:0] A, B;

output reg [15:0] leds;

output reg [5:0] out;

reg cin_reg, red_op_A_reg, red_op_B_reg, bypass_A_reg, bypass_B_reg, direction_reg, serial_in_reg;

reg [2:0] A_reg, B_reg;

reg [2:0] opcode_reg;

wire invalid, invalid_c1, invalid_c2;

assign invalid_c1 = (opcode_reg == 6 || opcode_reg == 7);

assign invalid_c2 = (red_op_A_reg || red_op_B_reg) && (opcode_reg != 0 && opcode_reg != 1);

assign invalid = invalid_c1 || invalid_c2;

always @(posedge clk or posedge rst) begin

if (rst) begin

out <= 0;

leds <= 0;

cin_reg <= 0;

red_op_A_reg <= 0;

red_op_B_reg <= 0;

bypass_A_reg <= 0;

bypass_B_reg <= 0;

direction_reg <= 0;

serial_in_reg <= 0;

opcode_reg <= 0;

A_reg <= 0;

B_reg <= 0;

end

else begin

if (invalid&&!(bypass_A_reg || bypass_B_reg)) leds <= ~leds;

else leds <= 0;

cin_reg <= cin;

red_op_A_reg <= red_op_A;

red_op_B_reg <= red_op_B;

bypass_A_reg <= bypass_A;

bypass_B_reg <= bypass_B;

direction_reg <= direction;

serial_in_reg <= serial_in;

opcode_reg <= opcode;

A_reg <= A;

B_reg <= B;

if (bypass_A_reg && bypass_B_reg)

out <= (INPUT_PRIORITY=="A")? A_reg:B_reg;

else if (bypass_A_reg)

out <= A_reg;

else if (bypass_B_reg)

out <= B_reg;

else if (invalid)

out <= 0;

else if (opcode_reg == 0) begin

if (red_op_A_reg && red_op_B_reg)

out <= (INPUT_PRIORITY=="A")? (|A_reg):(|B_reg);

else if (red_op_A_reg)

out <= |A_reg;

else if (red_op_B_reg)

out <= |B_reg;

else

out <= A_reg|B_reg;

end

else if (opcode_reg == 1) begin

if (red_op_A_reg && red_op_B_reg)

out <= (INPUT_PRIORITY=="A")? (^A_reg):(^B_reg);

else

out <= (red_op_A_reg)? (^A_reg): (red_op_B_reg)? (^B_reg):(A_reg^B_reg);

end

else if (opcode_reg == 2)

out <= (FULL_ADDER=="ON")? (A_reg + B_reg + cin_reg):(A_reg + B_reg);

else if (opcode_reg == 3)

out <= A_reg * B_reg;

else if (opcode_reg == 4) begin

if (direction_reg)

out <= {out[4:0], serial_in_reg};

else

out <= {serial_in_reg, out[5:1]};

end

else if (opcode_reg == 5) begin

if (direction_reg)

out <= {out[4:0],out[5]};

else

out <= {out[0],out[5:1]};

end

end

end

endmodule

/// ALSU after FIX

////////////////////////////////////////////////////////////////////////////////

// Author: Kareem Waseem

// Course: Digital Verification using SV & UVM

//

// Description: ALSU Design

//

////////////////////////////////////////////////////////////////////////////////

import shared_pkg::*;

module ALSU(A, B, cin, serial_in, red_op_A, red_op_B, opcode, bypass_A, bypass_B, clk, rst, direction, leds, out);

input clk, rst, cin, red_op_A, red_op_B, bypass_A, bypass_B, direction, serial_in;

input [2:0] opcode;

input signed [2:0] A, B;

output reg [15:0] leds;

output reg [5:0] out;

reg cin_reg, red_op_A_reg, red_op_B_reg, bypass_A_reg, bypass_B_reg, direction_reg, serial_in_reg;

reg [2:0] opcode_reg, A_reg, B_reg;

wire invalid_red_op, invalid_opcode, invalid;

assign invalid_red_op = (red_op_A_reg | red_op_B_reg) & (opcode_reg[1] | opcode_reg[2]);

assign invalid_opcode = opcode_reg[1] & opcode_reg[2]; // Fix

assign invalid = invalid_red_op | invalid_opcode;

always @(posedge clk or posedge rst) begin

if(rst) begin

leds <= 0;

out <= 0;

cin_reg <= 0;

red_op_B_reg <= 0;

red_op_A_reg <= 0;

bypass_B_reg <= 0;

bypass_A_reg <= 0;

direction_reg <= 0;

serial_in_reg <= 0;

opcode_reg <= 0;

A_reg <= 0;

B_reg <= 0;

end

else begin

if (invalid && !(bypass_A_reg || bypass_B_reg)) // FIX

leds <= ~leds;

else

leds <= 0;

cin_reg <= cin;

red_op_B_reg <= red_op_B;

red_op_A_reg <= red_op_A;

bypass_B_reg <= bypass_B;

bypass_A_reg <= bypass_A;

direction_reg <= direction;

serial_in_reg <= serial_in;

opcode_reg <= opcode;

A_reg <= A;

B_reg <= B;

if (bypass_A_reg && bypass_B_reg)

out <= (INPUT_PRIORITY == "A")? A_reg: B_reg;

else if (bypass_A_reg)

out <= A_reg;

else if (bypass_B_reg)

out <= B_reg;

else if (invalid)  // FIX

out <= 0; // FIX

else begin

case (opcode_reg) // FIX

3'h0: begin

if (red_op_A_reg && red_op_B_reg)

out = (INPUT_PRIORITY == "A")? |A_reg: |B_reg; // FIX

else if (red_op_A_reg)

out <= |A_reg; // FIX

else if (red_op_B_reg)

out <= |B_reg; // FIX

else

out <= A_reg | B_reg; // FIX

end

3'h1: begin

if (red_op_A_reg && red_op_B_reg)

out <= (INPUT_PRIORITY == "A")? ^A_reg: ^B_reg; // Fix

else if (red_op_A_reg)

out <= ^A_reg; // Fix

else if (red_op_B_reg)

out <= ^B_reg; // Fix

else

out <= A_reg ^ B_reg; // Fix

end

3'h2: begin

if (FULL_ADDER == "ON")

out <= A_reg + B_reg + cin_reg;

else

out <= A_reg + B_reg;

end

3'h3: out <= A_reg * B_reg;

3'h4: begin

if (direction_reg)

out <= {out[4:0], serial_in_reg};

else

out <= {serial_in_reg, out[5:1]};

end

3'h5: begin

if (direction_reg)

out <= {out[4:0], out[5]};

else

out <= {out[0], out[5:1]};

end

endcase

end

end

end

endmodule

/// assertion

import shared_pkg::*;

`define reset disable iff(rst)

module ALSU_sva(A, B, cin, serial_in, red_op_A, red_op_B, opcode, bypass_A, bypass_B, clk, rst, direction, leds, out);

input bit clk;

input logic rst, cin, red_op_A, red_op_B, bypass_A, bypass_B, direction, serial_in;

input [2:0] opcode;

input logic signed [2:0] A, B;

input logic [15:0] leds;

input logic [5:0] out;

////////////////////////////////////////////////////////

// this variable to make writing assertion more easer //

////////////////////////////////////////////////////////

bit isBypass, InvalidOP, bitwise;

assign isBypass = bypass_A | bypass_B;

assign InvalidOP = opcode==6 | opcode==7;

assign bitwise = opcode==0 | opcode==1;

assign isRed = red_op_B | red_op_A;

///////////////////////////////////

// assertion of bypass operation //

///////////////////////////////////

bypass_A_assert: assert property (@(posedge clk) `reset bypass_A |-> ##2 out==$past(A,2));

bypass_B_assert: assert property (@(posedge clk) `reset (bypass_B && !bypass_A) |-> ##2 out==$past(B,2));

//////////////////////////////////////

// assertion of reduction operation //

//////////////////////////////////////

sequence redValid(red, op,red2);

(red && !isBypass && opcode==op && !red2);

endsequence

redA_OR_assert: assert property (@(posedge clk) `reset redValid(red_op_A,0,0)        |-> ##2 out==|($past(A,2)));

redB_OR_assert: assert property (@(posedge clk) `reset redValid(red_op_B,0,red_op_A) |-> ##2 out==|($past(B,2)));

redA_XR_assert: assert property (@(posedge clk) `reset redValid(red_op_A,1,0)        |-> ##2 out==^($past(A,2)));

redB_XR_assert: assert property (@(posedge clk) `reset redValid(red_op_B,1,red_op_A) |-> ##2 out==^($past(B,2)));

////////////////////////////////

// assertion of invalid cases //

////////////////////////////////

sequence Invalid_seq;

((isRed && !isBypass && !bitwise) || (InvalidOP && !isBypass));

endsequence

Invalid_assert: assert property (@(posedge clk) `reset Invalid_seq |-> ##[0:2](out==0 && leds=='hffff));

////////////////////////////////////////

// assertion when opcode always valid //

////////////////////////////////////////

always_comb begin : assert_alwaysValid

if (alwaysValid) begin

alwValid_opcode_assert:  assert final (!InvalidOP);

alwZero_bypass_A_assert: assert final (!bypass_A);

alwZero_bypass_B_assert: assert final (!bypass_B);

alwZero_red_op_A_assert: assert final (!red_op_A);

alwZero_red_op_B_assert: assert final (!red_op_B);

end

end

/////////////////////

// reset assertion //

/////////////////////

always_comb begin : reset_assertion

if (rst) begin

rst_out_assert: assert final (out==0);

rst_leds_assert: assert final (leds==0);

end

end

//////////////////////////////////////////////////////////////////

// assertion for opcode Valid cases without bypass or reduction //

//////////////////////////////////////////////////////////////////

sequence sh_ro(op, dir);

(opcode==op && dir && !isRed && !isBypass);

endsequence

sequence op_assert(op);

(!isBypass && !isRed && opcode==op);

endsequence

OR_assert:   assert property (@(posedge clk) `reset op_assert(0) |-> ##2  out== $past(A,2) | $past(B,2) );

XOR_assert:  assert property (@(posedge clk) `reset op_assert(1) |-> ##2  out==($past(A,2)^$past(B,2)) );

add_assert:  assert property (@(posedge clk) `reset op_assert(2) |-> ##2  out==($past(A,2)+$past(B,2)+$past(cin,2)) );

mult_assert: assert property (@(posedge clk) `reset op_assert(3) |-> ##2  out==($past(A,2)*$past(B,2)) );

shiftL_assert: assert property (@(posedge clk) `reset sh_ro(4, direction) |-> ##2  out=={$past(out[4:0]), $past(serial_in,2)} );

shiftR_assert: assert property (@(posedge clk) `reset sh_ro(4, !direction) |-> ##2  out=={$past(serial_in,2), $past(out[5:1])} );

rotateL_assert: assert property (@(posedge clk) `reset sh_ro(5, direction) |-> ##2  out=={$past(out[4:0]), $past(out[5])} );

rotateR_assert: assert property (@(posedge clk) `reset sh_ro(5, !direction) |-> ##2  out=={$past(out[0]), $past(out[5:1])} );

/// Cover directives

/////////////////////////////////////////////////////////////////////////////

//////////////////////////          cover          //////////////////////////

/////////////////////////////////////////////////////////////////////////////

///////////////////////////////

// cover of bypass operation //

///////////////////////////////

bypass_A_cover: cover property (@(posedge clk) `reset bypass_A |-> ##2 out==$past(A,2));

bypass_B_cover: cover property (@(posedge clk) `reset (bypass_B && !bypass_A) |-> ##2 out==$past(B,2));

//////////////////////////////////

// cover of reduction operation //

//////////////////////////////////

redA_OR_cover: cover property (@(posedge clk) `reset redValid(red_op_A,0,0)        |-> ##2 out==|($past(A,2)));

redB_OR_cover: cover property (@(posedge clk) `reset redValid(red_op_B,0,red_op_A) |-> ##2 out==|($past(B,2)));

redA_XR_cover: cover property (@(posedge clk) `reset redValid(red_op_A,1,0)        |-> ##2 out==^($past(A,2)));

redB_XR_cover: cover property (@(posedge clk) `reset redValid(red_op_B,1,red_op_A) |-> ##2 out==^($past(B,2)));

////////////////////////////

// cover of invalid cases //

////////////////////////////

Invalid_cover: cover property (@(posedge clk) `reset Invalid_seq |-> ##[0:2](out==0 && leds=='hffff));

////////////////////////////////////

// cover when opcode always valid //

////////////////////////////////////

always_comb begin : cover_alwaysValid

if (alwaysValid) begin

alwValid_opcode_cover:  cover final (!InvalidOP);

alwZero_bypass_A_cover: cover final (!bypass_A);

alwZero_bypass_B_cover: cover final (!bypass_B);

alwZero_red_op_A_cover: cover final (!red_op_A);

alwZero_red_op_B_cover: cover final (!red_op_B);

end

end

/////////////////

// reset cover //

/////////////////

always_comb begin : reset_cover

if (rst) begin

rst_out_cover: cover final (out==0);

rst_leds_cover: cover final (leds==0);

end

end

//////////////////////////////////////////////////////////////

// cover for opcode Valid cases without bypass or reduction //

//////////////////////////////////////////////////////////////

OR_cover:   cover property (@(posedge clk) `reset op_assert(0) |-> ##2  out== $past(A,2) | $past(B,2) );

XOR_cover:  cover property (@(posedge clk) `reset op_assert(1) |-> ##2  out==($past(A,2) ^ $past(B,2)) );

add_cover:  cover property (@(posedge clk) `reset op_assert(2) |-> ##2  out==($past(A,2) + $past(B,2) + $past(cin,2)) );

mult_cover: cover property (@(posedge clk) `reset op_assert(3) |-> ##2  out==($past(A,2) * $past(B,2)) );

shiftL_cover: cover property (@(posedge clk) `reset sh_ro(4, direction) |-> ##2  out=={$past(out[4:0]), $past(serial_in,2)} );

shiftR_cover: cover property (@(posedge clk) `reset sh_ro(4, !direction) |-> ##2  out=={$past(serial_in,2), $past(out[5:1])} );

rotateL_cover: cover property (@(posedge clk) `reset sh_ro(5, direction) |-> ##2  out=={$past(out[4:0]), $past(out[5])} );

rotateR_cover: cover property (@(posedge clk) `reset sh_ro(5, !direction) |-> ##2  out=={$past(out[0]), $past(out[5:1])} );

endmodule

/// configuration

`timescale 1ps/1ps

package config_pkg;

import shared_pkg::*;

import uvm_pkg::*;

`include "uvm_macros.svh"

// Configration class

class ALSU_config extends uvm_object;

`uvm_object_utils(ALSU_config)

virtual ALSU_interface v_if;

function new(string name = "ALSU_config");

super.new(name);

endfunction //new()

endclass //ALSU_config

endpackage

/// sequenceItem (Valid and Invalid)

package sequenceItem_pkg;

import shared_pkg::*;

import uvm_pkg::*;

`include "uvm_macros.svh"

`define create_obj(type, name) type::type_id::create(name, this);

// Sequence Item class Valid and Invalid

class ALSU_sequenceItem extends uvm_sequence_item;

`uvm_object_utils(ALSU_sequenceItem)

logic clk;

rand logic rst, cin, red_op_A, red_op_B, bypass_A, bypass_B, direction, serial_in;

rand opcode_e opcode;

rand logic signed [2:0] A, B;

logic [15:0] leds;

logic [5:0] out;

logic [15:0] leds_ref;

logic [5:0] out_ref;

function new(string name = "ALSU_sequenceItem");

super.new(name);

endfunction //new()

// built in function

function string convert2string();

return $sformatf("%s, rst = %0d, A = %0d, B = %0d cin = %0d, red_op_A = %0d, red_op_B = %0d, bypass_A = %0d, bypass_B = %0d,

direction = %0d, serial_in = %0d \n out = %0d, out_ref = 0%0d ",

super.convert2string(), rst, A, B, cin, red_op_A, red_op_B, bypass_A, bypass_B, direction, serial_in, out, out_ref

);

endfunction

// my function to print ALSU inputs and outputs

function string convert2string_stim();

return $sformatf("rst = %0d, opcode = %0s, cin = %0d, red_op_A = %0d, red_op_B = %0d, bypass_A = %0d, bypass_B = %0d,

direction = %0d, serial_in = %0d \n out = %0d",

rst, opcode, cin, red_op_A, red_op_B, bypass_A, bypass_B, direction, serial_in, out

);

endfunction

/////// ////// ////// ////// //////

//       Constraint block        //

////// /////// ///// /////// //////

constraint rules1_7 {

//rst constraint

rst dist {0:=100-RESET_DIST, 1:=RESET_DIST};

// Invalid cases constraint

opcode dist {INVALID_6:=5, INVALID_7:=5, [0:5]:/90};

// A & B constraint when opcode is ADD or MULT

(opcode == MULT || opcode == ADD) -> A dist {MAXPOS:=20, ZERO:=10, MAXNEG:=20, [MAXNEG+1:MAXPOS-1]:/50};

(opcode == MULT || opcode == ADD) -> B dist {MAXPOS:=20, ZERO:=10, MAXNEG:=20, [MAXNEG+1:MAXPOS-1]:/50};

// A & B constraint when opcode is OR or XOR and red_op_A is high

((opcode==XOR || opcode==OR) && red_op_A) -> A dist {3'b001:=30, 3'b010:=30, 3'b100:=30, [MAXNEG+1:MAXPOS-1]:/10};

((opcode==XOR || opcode==OR) && red_op_A) -> B == 0;

// A & B constraint when opcode is OR or XOR and red_op_B is high

((opcode==XOR || opcode==OR) && red_op_B) -> A == 0;

((opcode==XOR || opcode==OR) && red_op_B) -> B dist {3'b001:=30, 3'b010:=30, 3'b100:=30, [MAXNEG+1:MAXPOS-1]:/10};

//  Do not constraint the inputs A or B when the operation is shift or rotate

// it's achieved by default after the 2,3 and 4 Constraint achieved

// bypass constraint

bypass_A dist {0:=90, 1:=10};

bypass_B dist {0:=90, 1:=10};

// red_op constraint

red_op_A dist {0:=80, 1:=20};

red_op_B dist {0:=80, 1:=20};

// A & B constraint when opcode is OR or XOR and red_op is low

((opcode==XOR || opcode==OR) && ~red_op_A && ~red_op_B) -> A == ~B;

}

rand opcode_e arr [6];

constraint rules_8 {

foreach(arr[i])

arr[i] inside {OR, XOR, ADD, MULT, SHIFT, ROTATE};

unique {arr};

//foreach(arr[i])

}

constraint MAKE_bypass_red_rst_ZERO {

rst == 0;

bypass_A == 0; bypass_B == 0;

red_op_A == 0; red_op_B == 0;

}

constraint INPUT_PRIORITY_CONS {

red_op_A == red_op_B;

bypass_A == bypass_B;

}

endclass //ALSU_sequenceItem extends uvm_sequence_item

/// sequenceItem (Valid Only)

package sequenceItem_valid_pkg;

import sequenceItem_pkg::*;

import shared_pkg::*;

import uvm_pkg::*;

`include "uvm_macros.svh"

`define create_obj(type, name) type::type_id::create(name, this);

// Sequence Item class Valid Cases Only

class ALSU_sequenceItem_valid extends ALSU_sequenceItem;

`uvm_object_utils(ALSU_sequenceItem_valid)

function new(string name = "ALSU_sequenceItem_valid");

super.new(name);

endfunction //new()

/////// ////// ////// ////// //////

//       Constraint block        //

////// /////// ///// /////// //////

constraint rules1_7 {

//rst constraint

rst dist {0:=100-RESET_DIST, 1:=RESET_DIST};

// make opcode always valid

//opcode dist {INVALID_6:=5, INVALID_7:=5, [0:5]:/90};

opcode inside {OR, XOR, ADD, MULT, SHIFT, ROTATE};

// to avoid invalid cases

if (opcode==OR||opcode==XOR)

red_op_A dist {0:=90, 1:=10};

else

red_op_A == 0;

if (opcode==OR||opcode==XOR)

red_op_B dist {0:=90, 1:=10};

else

red_op_B == 0;

// A & B constraint when opcode is ADD or MULT

(opcode == MULT || opcode == ADD) -> A dist {MAXPOS:=20, ZERO:=10, MAXNEG:=20, [MAXNEG+1:MAXPOS-1]:/50};

(opcode == MULT || opcode == ADD) -> B dist {MAXPOS:=20, ZERO:=10, MAXNEG:=20, [MAXNEG+1:MAXPOS-1]:/50};

// A & B constraint when opcode is OR or XOR and red_op_A is high

((opcode==XOR || opcode==OR) && red_op_A) -> A dist {3'b001:=30, 3'b010:=30, 3'b100:=30, [MAXNEG+1:MAXPOS-1]:/10};

((opcode==XOR || opcode==OR) && red_op_A) -> B == 0;

// A & B constraint when opcode is OR or XOR and red_op_B is high

((opcode==XOR || opcode==OR) && red_op_B) -> A == 0;

((opcode==XOR || opcode==OR) && red_op_B) -> B dist {3'b001:=30, 3'b010:=30, 3'b100:=30, [MAXNEG+1:MAXPOS-1]:/10};

//  Do not constraint the inputs A or B when the operation is shift or rotate

// it's achieved by default after the 2,3 and 4 Constraint achieved

// bypass constraint

bypass_A dist {0:=90, 1:=10};

bypass_B dist {0:=90, 1:=10};

// A & B constraint when opcode is OR or XOR and red_op is low

((opcode==XOR || opcode==OR) && ~red_op_A && ~red_op_B) -> A == ~B;

}

endclass //ALSU_sequenceItem_valid extends uvm_sequence_item

endpackage

/// ALSU_reset_sequence

`timescale 1ps/1ps

package rst_sequence_pkg;

import shared_pkg::*;

import sequenceItem_pkg::*;

import uvm_pkg::*;

`include "uvm_macros.svh"

`define create_obj(type, name) type::type_id::create(name);

class ALSU_reset_sequence extends uvm_sequence #(ALSU_sequenceItem);

`uvm_object_utils(ALSU_reset_sequence)

function new(string name = "ALSU_reset_sequence");

super.new(name);

endfunction //new()

ALSU_sequenceItem item;

// Main task

task body;

// Creat seq_item

item = `create_obj(ALSU_sequenceItem, "item")

item.A = 0; item.B = 0; item.opcode = OR;

item.cin = 0; item.serial_in = 0; item.direction = 0;

item.bypass_A = 0; item.bypass_B = 0;

item.red_op_A = 0; item.red_op_B = 0;

start_item(item);

item.rst = 1;

finish_item(item);

endtask

endclass //ALSU_reset_sequence extends uvm_sequence #(ALSU_sequenceItem)

endpackage

/// ALSU_main_sequence

`timescale 1ps/1ps

package main_sequence_pkg;

import shared_pkg::*;

import uvm_pkg::*;

import sequenceItem_pkg::*;

`include "uvm_macros.svh"

`define create_obj(type, name) type::type_id::create(name);

class ALSU_main_sequence extends uvm_sequence #(ALSU_sequenceItem);

`uvm_object_utils(ALSU_main_sequence)

function new(string name = "ALSU_main_sequence");

super.new(name);

endfunction //new()

ALSU_sequenceItem item;

// Main task

task body();

repeat(GIANT_LOOP) begin

item = `create_obj(ALSU_sequenceItem, "item")  // Creat seq_item

// edit constraint mode

item.constraint_mode(0);

item.rules1_7.constraint_mode(1);

start_item(item);

assert (item.randomize());

finish_item(item);

end

repeat(HOBBIT_LOOP) begin

item = `create_obj(ALSU_sequenceItem, "item")  // Creat seq_item

// edit constraint mode

item.constraint_mode(0);

item.INPUT_PRIORITY_CONS.constraint_mode(1);

start_item(item);

assert (item.randomize());

finish_item(item);

end

// make opcode onley valid

alwaysValid = 1;

repeat(DWARF_LOOP) begin

item = `create_obj(ALSU_sequenceItem, "item")  // Creat seq_item

// edit constraint mode

item.constraint_mode(0);

item.rules_8.constraint_mode(1);

item.MAKE_bypass_red_rst_ZERO.constraint_mode(1);

// randomization

assert (item.randomize());

// loop for opcode

foreach(item.arr[i]) begin

start_item(item);

item.opcode = item.arr[i];

finish_item(item);

end

end

endtask

endclass //ALSU_main_sequence extends uvm_sequence #(ALSU_sequenceItem)

endpackage

/// driver

`timescale 1ps/1ps

package driver_pkg;

import shared_pkg::*;

import sequenceItem_pkg::*;

import uvm_pkg::*;

`include "uvm_macros.svh"

`define create_obj(type, name) type::type_id::create(name);

// driver class

class ALSU_driver extends uvm_driver #(ALSU_sequenceItem);

`uvm_component_utils(ALSU_driver)

virtual ALSU_interface v_if;

ALSU_sequenceItem stim_seq_item;

function new(string name = "ALSU_driver", uvm_component parent = null);

super.new(name, parent);

endfunction //new()

task run_phase(uvm_phase phase);

super.run_phase(phase);

forever begin

//v_if.inputs_are_driven = 0;

stim_seq_item = `create_obj(ALSU_sequenceItem, "stim_seq_item")

seq_item_port.get_next_item(stim_seq_item);

// assigned inputs to interface

v_if.A = stim_seq_item.A;

v_if.B = stim_seq_item.B;

v_if.cin = stim_seq_item.cin;

v_if.serial_in = stim_seq_item.serial_in;

v_if.red_op_A = stim_seq_item.red_op_A;

v_if.red_op_B = stim_seq_item.red_op_B;

v_if.bypass_A = stim_seq_item.bypass_A;

v_if.bypass_B = stim_seq_item.bypass_B;

v_if.clk = stim_seq_item.clk;

v_if.rst = stim_seq_item.rst;

v_if.direction = stim_seq_item.direction;

// assigned opcode tp interface

v_if.opcode = stim_seq_item.opcode;

v_if.inputs_are_driven = 0;

@(negedge v_if.clk);

// assigned outputs to driver

stim_seq_item.leds = v_if.leds;

stim_seq_item.out = v_if.out;

// assigned refrence to driver

stim_seq_item.leds_ref = v_if.leds_ref;

stim_seq_item.out_ref = v_if.out_ref;

seq_item_port.item_done();

v_if.inputs_are_driven = 1;

`uvm_info("run_phase_driver", stim_seq_item.convert2string_stim(), UVM_HIGH)

end

endtask //run_phase

endclass //ALSU_driver extends uvm_driver

endpackage

/// sequencer

package sequencer_pkg;

import uvm_pkg::*;

import sequenceItem_pkg::*;

`include "uvm_macros.svh"

`define create_obj(type, name) type::type_id::create(name, this);

// sequencer class

class sequencer extends uvm_sequencer #(ALSU_sequenceItem);

`uvm_component_utils(sequencer)

function new(string name = "sequencer", uvm_component parent = null);

super.new(name, parent);

endfunction //new()

endclass //sequencer extends uvm_sequencer #(ALSU_sequenceItem)

endpackage

/// monitor

package monitor_pkg;

import shared_pkg::*;

import sequenceItem_pkg::*;

import uvm_pkg::*;

`include "uvm_macros.svh"

`define create_obj(type, name) type::type_id::create(name);

// monitor

class ALSU_monitor extends uvm_monitor;

`uvm_component_utils(ALSU_monitor)

virtual ALSU_interface v_if;

ALSU_sequenceItem mon_seq_item;

uvm_analysis_port #(ALSU_sequenceItem) mon_port; // monitor is a port

function new(string name = "ALSU_monitor", uvm_component parent = null);

super.new(name, parent);

endfunction //new()

function void build_phase(uvm_phase phase);

super.build_phase(phase);

mon_port = new("mon_port", this);

endfunction

task run_phase(uvm_phase phase);

super.run_phase(phase);

forever begin

mon_seq_item = `create_obj(ALSU_sequenceItem, "mon_seq_item")

@(negedge v_if.clk);

// assigned inputs to monitor

mon_seq_item.A = v_if.A;

mon_seq_item.B = v_if.B;

mon_seq_item.cin = v_if.cin;

mon_seq_item.serial_in = v_if.serial_in;

mon_seq_item.red_op_A = v_if.red_op_A;

mon_seq_item.red_op_B = v_if.red_op_B;

mon_seq_item.bypass_A = v_if.bypass_A;

mon_seq_item.bypass_B = v_if.bypass_B;

mon_seq_item.clk = v_if.clk;

mon_seq_item.rst = v_if.rst;

mon_seq_item.direction = v_if.direction;

// assigned outputs to monitor

mon_seq_item.leds = v_if.leds;

mon_seq_item.out = v_if.out;

// assigned outputs to monitor

mon_seq_item.leds_ref = v_if.leds_ref;

mon_seq_item.out_ref = v_if.out_ref;

// assigned opcode tp monitor

mon_seq_item.opcode = opcode_e'(v_if.opcode);

mon_port.write(mon_seq_item); // that's mean that monitor will send the data

`uvm_info("run_phase_monitor", mon_seq_item.convert2string_stim(), UVM_FULL)

end

endtask //run_pha

endclass //ALSU_monitor extends uvm_monitor

endpackage

/// agent

package agent_pkg;

import shared_pkg::*;

import config_pkg::*;

import driver_pkg::*;

import sequencer_pkg::*;

import sequenceItem_pkg::*;

import monitor_pkg::*;

import uvm_pkg::*;

`include "uvm_macros.svh"

`define create_obj(type, name) type::type_id::create(name, this);

// agent class

class ALSU_agent extends uvm_agent;

`uvm_component_utils(ALSU_agent)

sequencer sqr; // mange data transfer

ALSU_driver drv; // inside agent

ALSU_monitor mon; // inside agent

ALSU_config cfg; // get the data of interface

uvm_analysis_port #(ALSU_sequenceItem) agt_port; // agent is a port

function new(string name = "ALSU_agent", uvm_component parent = null);

super.new(name, parent);

endfunction //new()

function void build_phase(uvm_phase phase);

super.build_phase(phase);

if (!uvm_config_db#(ALSU_config)::get(this, "", "CFG", cfg))

`uvm_fatal("build_phase", "DRIVER - Unable to get config");

sqr = `create_obj(sequencer, "sqr")

drv = `create_obj(ALSU_driver, "drv")

mon = `create_obj(ALSU_monitor, "mon")

agt_port = new("agt_port", this);

endfunction

function void connect_phase(uvm_phase phase);

drv.v_if = cfg.v_if;

mon.v_if = cfg.v_if;

drv.seq_item_port.connect(sqr.seq_item_export);

mon.mon_port.connect(agt_port);

endfunction //connect_phase

endclass //ALSU_agent extends uvm_agent

endpackage

/// coverage_collector

package coverage_collector_pkg;

import agent_pkg::*;

import shared_pkg::*;

import sequencer_pkg::*;

import sequenceItem_pkg::*;

import uvm_pkg::*;

`include "uvm_macros.svh"

class ALSU_coverage extends uvm_component;

`uvm_component_utils(ALSU_coverage)

uvm_analysis_export #(ALSU_sequenceItem) cov_export; // coverage export

uvm_tlm_analysis_fifo #(ALSU_sequenceItem) cov_fifo; // coverage fifo

ALSU_sequenceItem cov_seq_item;

//////////////////////////////////

//      begin Coverage Group    //

//////////////////////////////////

covergroup cvr_gp;

// input A bins

A_cp: coverpoint cov_seq_item.A {

bins A_data_0 = {ZERO};

bins A_data_max = {MAXPOS};

bins A_data_min = {MAXNEG};

bins A_data_default = default;

bins A_data_[] = {001, 010, 100};

}

// input B bins

B_cp: coverpoint cov_seq_item.B {

bins B_data_0 = {ZERO};

bins B_data_max = {MAXPOS};

bins B_data_min = {MAXNEG};

bins B_data_default = default;

bins B_data_[] = {001, 010, 100};

}

// cover point for reduction operation red_op

op_A_cp: coverpoint cov_seq_item.red_op_A {

bins one = {1};

bins zero = {0};

}

op_B_cp: coverpoint cov_seq_item.red_op_B {

bins one = {1};

bins zero = {0};

}

// Crossing to satsfied the data_walkingones of A and B

A_walk: cross A_cp, op_A_cp {

option.cross_auto_bin_max = 0;

bins A_data_walkingones = binsof(A_cp.A_data_) && binsof(op_A_cp.one);

}

B_walk: cross B_cp, op_A_cp, op_B_cp {

option.cross_auto_bin_max = 0;

bins B_data_walkingones = (binsof(B_cp.B_data_)

&& binsof(op_A_cp.zero)

&& binsof(op_B_cp.one));

}

// cover point for opcode (ALU)

ALU_cp: coverpoint cov_seq_item.opcode {

bins Bins_shift[] = {SHIFT, ROTATE};

bins Bins_arith[] = {ADD, MULT};

bins Bins_bitwise[] = {OR, XOR};//

bins Bins_invalid = {INVALID_6, INVALID_7};

bins Bins_trans = (0 => 1 => 2 => 3 => 4 => 5);

}

// cover point for c_in

cin_cp: coverpoint cov_seq_item.cin;

// cover point for serial_in

serial_cp: coverpoint cov_seq_item.serial_in;

// cover point for direction

direction_cp: coverpoint cov_seq_item.direction;

// Cross coverage between ALU_cp and A and B

ALU_A: cross ALU_cp, A_cp {

option.cross_auto_bin_max = 0;

bins arith_permutations = binsof(ALU_cp.Bins_arith) && binsof(A_cp) intersect{ZERO, MAXPOS, MAXNEG};

}

ALU_B: cross ALU_cp, B_cp {

option.cross_auto_bin_max = 0;

bins arith_permutations = binsof(ALU_cp.Bins_arith) && binsof(B_cp) intersect{ZERO, MAXPOS, MAXNEG};

}

// Cross coverage between ALU_cp and cin_cp

ALU_cin: cross ALU_cp, cin_cp {

option.cross_auto_bin_max = 0;

bins add_cin = binsof(ALU_cp) intersect{ADD};

}

// Cross coverage between ALU_cp and serial_cp

ALU_serial: cross ALU_cp, serial_cp {

option.cross_auto_bin_max = 0;

bins shift_serial = binsof(ALU_cp) intersect{SHIFT};

}

// Cross coverage between ALU_cp and direction_cp

ALU_direction: cross ALU_cp, direction_cp {

option.cross_auto_bin_max = 0;

bins sh_ro_direction = binsof(ALU_cp.Bins_shift);

}

// Cross coverage ALU = {OR,XOR}, red_op_A = 1, A = data_walk, B = 0

A_data_walk_OR_XOR: cross ALU_cp, A_walk, op_A_cp, B_cp {

option.cross_auto_bin_max = 0;

bins A_walk_OR_XOR = (binsof(ALU_cp.Bins_bitwise)

&& binsof(A_walk.A_data_walkingones)

&& binsof(op_A_cp.one)

&& binsof(B_cp.B_data_0));

}

// Cross coverage ALU = {OR,XOR}, red_op_A = 1, A = data_walk, B = 0

B_data_walk_OR_XOR: cross ALU_cp, B_walk, op_B_cp, A_cp {

option.cross_auto_bin_max = 0;

bins B_walk_OR_XOR = (binsof(ALU_cp.Bins_bitwise)

&& binsof(B_walk.B_data_walkingones)

&& binsof(op_B_cp.one)

&& binsof(A_cp.A_data_0));

}

// Cross coverage Invalid case 2 red_op

Invalid_red_op: cross ALU_cp, op_A_cp, op_B_cp {

option.cross_auto_bin_max = 0;

bins Invalid_reduction = (binsof(ALU_cp.Bins_bitwise)

&& (binsof(op_A_cp.one) || binsof(op_B_cp.one)));

}

// Invalid case with reduction operation

reduction_invalid: cross ALU_cp, op_A_cp, op_B_cp {

option.cross_auto_bin_max = 0;

bins invalid_red_op = (binsof(ALU_cp) intersect{!OR, !XOR} && (binsof(op_A_cp.one)||binsof(op_B_cp.one)));

}

// cover point rst

rst_cp: coverpoint cov_seq_item.rst;

// Cross coverage red_op_A and red_op_B

red_op_High_cross: cross op_A_cp, op_B_cp;

endgroup

///////////////////////////////////

//      finish Coverage Group    //

///////////////////////////////////

function new(string name = "ALSU_coverage", uvm_component parent = null);

super.new(name, parent);

cvr_gp = new();

endfunction //new()

function void build_phase(uvm_phase phase);

super.build_phase(phase);

cov_export = new("cov_export", this);

cov_fifo = new("cov_fifo", this);

endfunction

function void connect_phase(uvm_phase phase);

super.connect_phase(phase);

cov_export.connect(cov_fifo.analysis_export);

endfunction

task run_phase(uvm_phase phase);

super.run_phase(phase);

forever begin

cov_fifo.get(cov_seq_item);

cvr_gp.sample();

end

endtask

endclass //ALSU_coverage extends uvm_component

endpackage

/// scoreboard

package scoreboard_pkg;

import agent_pkg::*;

import shared_pkg::*;

import sequenceItem_pkg::*;

import uvm_pkg::*;

`include "uvm_macros.svh"

`define create_obj(type, name) type::type_id::create(name, this);

class ALSU_scoreboard extends uvm_scoreboard;

`uvm_component_utils(ALSU_scoreboard)

uvm_analysis_export #(ALSU_sequenceItem) sb_export; // scoreboard export

uvm_tlm_analysis_fifo #(ALSU_sequenceItem) sb_fifo; // scoreboard fifo

ALSU_sequenceItem sb_seq_item;

// refrence output

logic [15:0] leds_ref;

logic [5:0] out_ref;

// error and correct counter

int correct_counter = 0;

int error_counter = 0;

function new(string name = "ALSU_scoreboard", uvm_component parent = null);

super.new(name, parent);

endfunction //new()

function void build_phase(uvm_phase phase);

super.build_phase(phase);

sb_export = new("sb_export", this);

sb_fifo = new("sb_fifo", this);

endfunction

function void connect_phase(uvm_phase phase);

super.connect_phase(phase);

sb_export.connect(sb_fifo.analysis_export);

endfunction

task run_phase(uvm_phase phase);

super.run_phase(phase);

forever begin

sb_fifo.get(sb_seq_item);

//// Checking //////

Checking_task(sb_seq_item);

end

endtask

task Checking_task(ALSU_sequenceItem chk_item);

if (chk_item.out != chk_item.out_ref || chk_item.leds != chk_item.leds_ref) begin

error_counter++;

`uvm_error("scoreboard",$sformatf("%0s\nout_ref = %0d,",chk_item.convert2string(), chk_item.out_ref))

end else begin

correct_counter++;

end

endtask //Checking_task

function void report_phase(uvm_phase phase);

super.report_phase(phase);

`uvm_info("report_phase", $sformatf("Total correct transaction: %0d", correct_counter), UVM_LOW)

`uvm_info("report_phase", $sformatf("Total faild transaction: %0d", error_counter), UVM_LOW)

endfunction

endclass //ALSU_scoreboard extends uvm_scoreboard

endpackage

/// env

`timescale 1ps/1ps

package env_pkg;

import shared_pkg::*;

import scoreboard_pkg::*;

import coverage_collector_pkg::*;

import agent_pkg::*;

import uvm_pkg::*;

`include "uvm_macros.svh"

`define create_obj(type, name) type::type_id::create(name, this);

// Environment class

class ALSU_env extends uvm_env;

`uvm_component_utils(ALSU_env)

ALSU_scoreboard sb;

ALSU_coverage cov;

ALSU_agent agt;

// declare new() function of parent uvm_env

function new(string name = "ALSU_env", uvm_component parent = null);

super.new(name, parent);

endfunction //new()

// build phase function and send the parameter phase to parent uvm_env

function void build_phase(uvm_phase phase);

super.build_phase(phase);

// create scoreboard, coverage and agent

// if we have more than one I should change name (first parameter)

agt = `create_obj(ALSU_agent, "agt")

sb = `create_obj(ALSU_scoreboard, "sb")

cov = `create_obj(ALSU_coverage, "cov")

// Override the default instantiation of ALSU_sequenceItem objects with ALSU_sequenceItem_valid objects

//uvm_factory::set_default_create_ "ALSU_sequenceItem", () => ALSU_sequenceItem_valid::type_id::create();

endfunction

function void connect_phase(uvm_phase phase);

agt.agt_port.connect(sb.sb_export);   // why we connect with export directly why not fifo first

agt.agt_port.connect(cov.cov_export); // why we connect with export directly why not fifo first

endfunction

endclass //ALSU_env extends uvm_env

endpackage

/// test

package test_pkg;

import shared_pkg::*;

import env_pkg::*;

import config_pkg::*;

import main_sequence_pkg::*;

import rst_sequence_pkg::*;

import sequenceItem_pkg::*;

import sequenceItem_valid_pkg::*;

import uvm_pkg::*;

`include "uvm_macros.svh"

`define create_obj(type, name) type::type_id::create(name, this);

class ALSU_test extends uvm_test;

`uvm_component_utils(ALSU_test)

ALSU_env env;

ALSU_config cfg;

ALSU_reset_sequence rst_seq;

ALSU_main_sequence main_seq;

// declare new() function of parent uvm_test

function new(string name = "ALSU_test", uvm_component parent = null);

super.new(name, parent);

endfunction //new()

// build phase function and send the parameter phase to parent uvm_test

function void build_phase(uvm_phase phase);

uvm_factory factory = uvm_factory::get();

super.build_phase(phase);

set_type_override_by_type(ALSU_sequenceItem::get_type(), ALSU_sequenceItem_valid::get_type());

factory.print();

env = `create_obj(ALSU_env, "env");

cfg = `create_obj(ALSU_config, "cfg");

rst_seq = `create_obj(ALSU_reset_sequence, "rst_seq")

main_seq = `create_obj(ALSU_main_sequence, "main_seq")

if (!uvm_config_db#(virtual ALSU_interface)::get(this, "", "INTERFACE", cfg.v_if))

`uvm_fatal("build_phase", "TEST - Unable to get config");

uvm_config_db#(ALSU_config)::set(this, "*", "CFG", cfg);

endfunction

// run phase function to create UVM env

task run_phase(uvm_phase phase);

super.run_phase(phase);

// raise and drop to start and finish of ALSU_test

phase.raise_objection(this);

// ONLY SHOW IF +UVM_VERBOSITY=UVM_DEBUG

#1; `uvm_info("run_phase", "Inside the slaby test DEBUG", UVM_DEBUG)

////////////////////

//    stimulus    //

////////////////////

// rst seq

`uvm_info("run_phase", "ALSU reset seq start", UVM_MEDIUM)

rst_seq.start(env.agt.sqr);

`uvm_info("run_phase", "ALSU reset seq finish", UVM_MEDIUM)

// Main seq

`uvm_info("run_phase", "ALSU main seq start", UVM_MEDIUM)

main_seq.start(env.agt.sqr);

`uvm_info("run_phase", "ALSU reset seq finish", UVM_MEDIUM)

phase.drop_objection(this);

endtask

endclass //ALSU_test extends uvm_test

endpackage

/// top

`timescale 1ps/1ps

import uvm_pkg::*;

`include "uvm_macros.svh"

import test_pkg::*;

module top ();

bit clk;

initial begin

forever #1 clk = ~clk;

end

ALSU_interface intf (clk);

ALSU DUT (

intf.A, intf.B, intf.cin, intf.serial_in, intf.red_op_A,

intf.red_op_B, intf.opcode, intf.bypass_A, intf.bypass_B,

intf.clk, intf.rst, intf.direction, intf.leds, intf.out

);

ALSU_ref GLD (

intf.A, intf.B, intf.cin, intf.serial_in, intf.red_op_A,

intf.red_op_B, intf.opcode, intf.bypass_A, intf.bypass_B,

intf.clk, intf.rst, intf.direction, intf.leds_ref, intf.out_ref

);

bind ALSU ALSU_sva ALSU_sva_inst(

A, B, cin, serial_in, red_op_A, red_op_B,

opcode, bypass_A, bypass_B, clk, rst, direction, leds, out

);

initial begin

uvm_config_db#(virtual ALSU_interface)::set(null, "uvm_test_top", "INTERFACE", intf);

run_test("ALSU_test");

end

endmodule

2. Simulate the top module. Take a screenshot of the UVM report to use it in your PDF.

- UVM Report

- Constraint of rule 8  make opcode only valid and unique values

- Constraint reset most of the time is inactive

- Invalid reduction

- Invalid opcode

| - OR | - XOR | - ADD |
| --- | --- | --- |
| - MULT | - SHIFT | - ROTATE |

3. Add the signals of the interface to wave and make sure that the inputs are driven. Take a screenshot and add it to your PDF.

Do file:

add wave -position insertpoint sim:/top/intf/*

4. Take screenshots of the code coverage, functional coverage, sequential domain (assertions) coverage for ALSU

- Assertion

- Coverage directives

- Coverage

1. Questa Group Coverage

2. Functional coverage

3. Code coverage

- HTML report

- Txt report

  - Branch Coverage

  - Condition Coverage

  - Expression Coverage

  - Statement Coverage

  - Toggle Coverage

4. Sequential domain coverage

///// using factory property to override only valid sequencs, Leds never get blink/////

/// All cover directives were covered but the Invalid one.

////   Code

function void build_phase(uvm_phase phase);

uvm_factory factory = uvm_factory::get();

super.build_phase(phase);

set_type_override_by_type(ALSU_sequenceItem::get_type(), ALSU_sequenceItem_valid::get_type());

factory.print();

/// Factory report

///  without the override method call, Leds blink then No blink (Valid and Invalid cases) ///

/// Invalid cases was covered

/// Code

function void build_phase(uvm_phase phase);

uvm_factory factory = uvm_factory::get();

super.build_phase(phase);

//set_type_override_by_type(ALSU_sequenceItem::get_type(), ALSU_sequenceItem_valid::get_type());

factory.print();

/// Factory report